PAGE  
Page 1 of 2

Clements Chemicals
SAFETY DATA SHEET
PREWASH
Updated:  23 Jul. 03

Date of Printing:  22 Mar. 12
MSDS Revision:  01
1.  IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND COMPANY/UNDERTAKING






Product Name:


Prewash

Product NO:  


B773, A869
Use of the substance/preparation:
Laundry prewash powder

Supplier:



William Clements (Chemicals) Ltd





The Old Transport Museum






Witham Street






BELFAST BT4 1HP


Telephone:


(028) 9073 8395

2.  COMPOSITION/INFORMATION ON INGREDIENTS









	Ingredient Name
	Cas No
	Contents (%)
	Health (class)
	Risk (R No.)

	Sodium Metasilicate
	6834-92-0
	>50
	C
	34


3.  HAZARDS IDENTIFICATION











Alkaline granules.  Causes burns.  May cause permanent damage to eyes.  
4.  FIRST AID MEASURES












Inhalation:  
Irritation of nose, throat and lungs, coughing.  Remove from exposure, rest and keep warm.  In severe cases, or if recovery is not rapid or complete seek medical attention.  
Ingestion:
Burns to throat and upper digestive tract.  Wash out with water.  Do not induce vomiting.  If patient is conscious, give water to drink.  If patient feels unwell seek medical attention.  
Skin Contact: 
Burning sensation.  Drench the skin with plenty of water.  Remove contaminated clothing and wash before reuse.  If large s damaged or if irritation persists seek medical attention.  
Eye Contact:  
Severe pain, reddening.  Irrigate thoroughly with water for a t least 10 minutes.  Obtain medical attention.  
5.  FIRE-FIGHTING MEASURES











Suitable Extinguishing Media:

Use extinguisher suitable to cause of fire.

Protective equipment for fire-fighting:  
Wear protective  clothing.    
6.  ACCIDENTAL RELEASE MEASURES











Personal precautions: 
Wear appropriate PPE
Environmental precautions:  
If material enters drains or watercourses, dilute as much as possible and inform authorities.  

Spillages:  

Shovel up spilt material into suitable containers and hold for waste disposal.  
7.  HANDLING AND STORAGE











Usage Precautions:

Provide good general ventilation.  LEV may be required in dust environments.   
Storage Precautions:

Temperature range:  
Ambient
Humidity range:  

dry

Personal Precautions:  
Always wear protective clothing.  


8.  EXPOSURE CONTROLS/PERSONAL PROTECTION









Exposure limits:

2mg/m3 STEL  
Type suggested.   

Protective measures:

Respirators:

Type approved RPE for alkaline dusts
Protective Gloves:  

Impervious gloves
Eye Protection:  

Goggles or face shield
Other Protection:  

Overalls and boots
Hygiene measures: 

Always wash thoroughly after handling chemicals.  
9.  PHYSICAL AND CHEMICAL PROPERTIES










Appearance:
White granules
10.  STABILITY AND REACTIVITY











Known hazardous reactions:  
Solutions will react with aluminium, zinc, tin and their alloys to evolve hydrogen gas.  
11.  TOXICOLOGICAL INFORMATION










Effects:  
Harmful if swallowed, causing burns to mouth and throat.  Inhalation of dust will cause severe irritation to nose, throat and lungs.  Causes skin and eye burns.  May cause permanent damage to eyes if not treated promptly.  

LD50
1280 mg/Kg oral-rat  

12.  ECOLOGICAL INFORMATION











Environmental Effects:  

On dilution, material rapidly depolymerises to molecular species that are indistinguishable from natural dissolved silica.  However, the pH will be above acceptable limits for discharge.  
13.  DISPOSAL CONSIDERATIONS











Product Disposal:

Via an authorised waste disposal contractor to an approved waste disposal site, observing all 




local and national regulations
Container Disposal:  
As substance.  
14.  TRANSPORT INFORMATION











UNNO


1759
Primary Hazard


Corrosive
Packing Group


III

H.I. Number


80

Emergency Action Code

2X
15. REGULATORY INFORMATION











Label for Supply:

C
Corrosive

Risk Phrases:

34
Causes burns
Safety Phrases:  

2 
Keep out of the reach of children

26
In case of contact with eyes rinse immediately with plenty of water and seek medical advice

27
Take off immediately all contaminated clothing
37/39
Wear suitable gloves and eye/face protection

16.  OTHER INFORMATION












The above information is based on the present state of our knowledge of the product at the time of publication.  It is given in good faith, no warranty is implied with respect to the quality or the specification of the product.  The use must satisfy himself that the product s entirely suitable for his purpose. 
